## N714D Annual Condition Inspection Checklist

### General

- Note hobbs & tach time

- Check ELT

### Main Landing Gear

- Remove wheel pants

- Inspect tires

- Inspect main landing gear

- Inspect brake lines and calipers

- Remove wheels and tires

- Install new brake linings (if necessary)

- Separate wheel halves, remove all bearing components

- Clean bearing components and inspect for pitting

- Repack wheel bearings with Aeroshell #5

- Reassemble wheels and tires, rotate tires (if necessary)

- Bleed brakes (if necessary)

- Inspect wheel pants and mounting hardware

- Inflate tires to 40 psi

- Reinstall wheel pants

### Tailwheel

- Inspect tailwheel

- Inspect tailwheel fork and spring

- Inspect tailwheel steering link

- Disassemble and clean tailwheel fork

- Inspect condition of rudder lock mechanism

- Rebuild tailwheel fork and lubricate with Aeroshell #5

- Lubricate tailwheel bearings with Aeroshell #5

- Rotate tailwheel (if necessary)

### Right wing

- Remove wing tip

- Remove lower access panels

- Remove wing root fairing

- Inspect NAV antenna and strobe/position light wires & connections

- Inspect exterior wing surfaces for cracks

- Inspect fuel tank exterior for leaks

- Inspect aileron exterior for cracks

- Inspect aileron hinge brackets

- Inspect aileron counterweight “pipe”

- Inspect flap exterior for cracks

- Inspect interior wing structure and interior of skins

- Inspect condition of pushrods

- Inspect bellcrank & pushrod ends for play and wear

- Inspect flap hinge & check pins secure

- Inspect fuel cap, rebuild/lubricate with EZ-Turn (if necessary)

- Inspect fuel tank interior

- Inspect fuel drain

- Inspect fuel tank root rib for leaks

- Inspect fuel and vent tubing and fittings

- Inspect fuel quantity sender & wiring

- Inspect front spar

- Inspect wing root area

- Inspect rear spar attach point

- Inspect landing light and lens

- Inspect OAT probe

- Inspect OAT wire and conduit

- Lubricate bellcrank & rod end bearings with LPS-2

- Lubricate aileron hinges with LPS-2

- Lubricate lower flap pushrod end bearing with LPS-2

- Actuate flaps and inspect through range of travel

- Reinstall wing tip

- Reinstall lower access panels

- Reinstall wing root fairing

### Left wing

- Remove wing tip

- Remove lower access panels

- Remove wing root fairing

- Inspect COM antenna and strobe/position light wires & connections

- Inspect exterior wing surfaces for cracks

- Inspect fuel tank exterior for leaks

- Inspect aileron exterior for cracks

- Inspect aileron hinge brackets

- Inspect aileron counterweight “pipe”

- Inspect flap exterior for cracks

- Inspect interior wing structure and interior of skins

- Inspect condition of pushrods

- Inspect bellcrank & pushrod ends for play and wear

- Inspect flap hinge & check pins secure

- Inspect fuel cap, rebuild/lubricate with EZ-Turn (if necessary)

- Inspect fuel tank interior

- Inspect fuel drain

- Inspect fuel tank root rib for leaks

- Inspect fuel and vent tubing and fittings

- Inspect fuel quantity sender & wiring

- Inspect front spar

- Inspect wing root area

- Inspect rear spar attach point

- Inspect landing light and lens

- Inspect autopilot servo, linkages, and connector

- Inspect pitot tubing, pitot heat wire, autopilot wire, conduit

- Lubricate bellcrank & rod end bearings with LPS-2

- Lubricate aileron hinges with LPS-2

- Lubricate lower flap pushrod end bearing with LPS-2

- Actuate flaps and inspect through range of travel

- Reinstall wing tip

- Reinstall lower access panels

- Reinstall wing root fairing

### Tailcone and Empennage

- Remove empennage fairing and tail inspection panels

- Inspect fuselage aft of F-708

- Inspect longerons, VS and HS spars, doublers, rivets

- Inspect tail spring mount weldment

- Lubricate elevator pushrod end and center elevator hinge with LPS-2

- Lubricate elevator hinge rod ends with LPS-2

- Lubricate rudder hinge rod ends with LPS-2

- Inspect rudder cable clevis ends and hardware

- Inspect rudder stops

- Inspect exterior of tail surfaces for cracks

- Inspect elevator & rudder counterweights for security

### Fuselage Exterior

- Inspect exterior skins and rivets for cracks

- Inspect static ports

- Clean belly

- Inspect external antennas

- Inspect fuel vents

- Inspect condition of rear window

- Tighten rear window screws (if necessary)

- Inspect canopy for cracks

### Fuselage Interior

- Remove seat cushions, seat backs, stick boots

- Remove F-706 baggage bulkhead panels

- Remove flap actuator cover panels

- Remove aft tunnel cover

- Remove flap housing cover panels

- Remove carpet

- Remove seat pans

- Remove fuel pump & filter housing

- Remove center cabin cover

- Remove spar cabin cover

- Inspect longerons, bulkheads, skins, etc.

- Inspect elevator bellcrank & pushrod ends for play and wear

- Inspect tailcone wiring & conduit

- Inspect static tubing

- Inspect ELT mount

- Inspect strobe power supply

- Inspect condition of cabin frame/roll bar

- Inspect elevator pushrod tunnel

- Inspect flap actuator

- Inspect flap position sensor and linkage

- Inspect spar and center section

- Inspect rear spar beneath seat pans

- Inspect trim and flap relays and wiring

- Inspect antenna connections

- Inspect control column, sticks, and pushrods

- Replace ELT batteries (if necessary)

- Lubricate flap pushrod upper ends with LPS-2

- Actuate flaps and inspect mechanism through range of travel

- Lubricate elevator pushrod ends and bellcrank with LPS-2

- Lubricate aileron pushrod end bearings

- Lubricate control column bearings

- Remove, inspect, clean, and reinstall fuel filter

- Inspect and pressure test fuel system

- Inspect pitot tubing

- Inspect brake system for leaks

- Inspect parking brake and cable

- Inspect rudder pedal weldments

- Inspect rudder cables and links

- Inspect instruments, avionics, and wiring

- Inspect manifold pressure tubing

- Inspect fuel vent tubing

- Inspect firewall and forward fuselage structure

- Reinstall all cover panels

- Reinstall cabin cover panels, seat pans, and flap housing covers

- Reinstall front deck access panels

- Reinstall carpet (replace with new if necessary)

- Inspect and oil canopy struts

- Inspect canopy latches

- Inspect canopy seals

### Firewall Forward

- Fly plane, warm up oil

- Note hobbs and tach time

- Remove cowling

- Drain oil & filter

- Remove top plugs

- Differential compression test:

  - #1:		/80

  - #2:		/80

  - #3:		/80

  - #4:		/80

- Check mag timing 20 degrees BTDC

- Check propeller blade track (while top spark plugs removed)

- Remove oil filter

- Cut open and inspect old oil filter

- Inspect top spark plug condition and gap to .034” or replace

- Remove bottom spark plugs

- Inspect bottom spark plugs, gap to .016” or replace

- Inspect alternator belt for condition and tension

- Inspect starter ring gear teeth

- Remove battery

- Remove and clean fuel injectors

- Remove air box and air filter

- Remove and inspect heat SCAT tubing

- Remove and inspect oil pickup screen

- Clean air filter with K&N air filter cleaner, let dry

- Clean engine

- Inspect engine controls

- Inspect exhaust for cracks

- Inspect heat muff

- Inspect engine mount for cracks

- Inspect brake lines along firewall

- Inspect firewall

- Inspect oil cooler

- Inspect baffles

- Inspect hoses and fittings

- Inspect brake fluid reservoir, check fluid level

- Torque intake bolts

- Torque exhaust bolts

- Lubricate engine controls with LPS-2

- Lubricate throttle shaft

- Install & safety wire new oil filter

- Fill sump with 7 ½ quarts of Aeroshell 15W50

- Service air filter with K&N air filter oil

- Reinstall air filter and air box

- Install new battery (if necessary)

- Run up engine, check for leaks

- Inspect top and bottom cowling and hinges

- Reinstall cowling

### Propeller

- Remove spinner

- Inspect spinner for cracks and wear

- Inspect spinner bulkheads for cracks

- Inspect propeller attach hardware and safety wire

- Inspect and clean propeller hub exterior

- Lubricate propeller hub with Aeroshell 6

- Clean propeller blades

- Inspect propeller blades for nicks and cracks

- File/smooth out nicks in propeller blades

- Paint propeller (if necessary)

- Reinstall spinner