RV-9A 90577 Panel Removal/Installation

- Remove and store any removable avionics and attachments

- Disconnect from back of EFIS:

- D-sub connector

- Pitot line

- Static line

- AOA line

- Disconnect from back of ASI:

- Pitot line

- Static line

- Disconnect:

- switch harness fast-on terminals

- ground harness fast-on terminals at firewall -- pull harness through two cushion clamps to remove.

- Main power connection

- Disconnect sub-panel connectors:

- Magneto (box connector)

- Dimmer (box connector)

- Autopilot (D-sub connector)

- Disconnect coax cables (BNC connectors):

- VHF COM1

- Transponder

- Traffic Monitor

- Portable NAV/COM (left side panel)

- Disconnect ELT

- Disconnect Autopilot at back of instrument (D-sub connector)

- Disconnect Audio circuit from back of panel (D-sub connector)

- Tie wrap loose cables to prevent snagging when panel is removed

- Unscrew side panels

- Unscrew main panel

- Remove panel from aircraft

- To install, reverse removal instructions